Custom electronics and home-brewing

Section 1 - custom electronics and Arduino

Reasons for needing custom panels

No off the shelf parts that fit exactly or is simple enough. Things like the "Big Knob" are great but are overly complex for most situations.

Making monitors intelligently switch their inputs, for example;

Differences between wet and dry GPIs - example; TriCaster has 5v low current tallies out whereas you may need to light your own 12v tally lamps

Wireless switching - NBC audio suite example.

Metalwork

A metal workshop will always make a better job of punching and engraving and will charge less than you think.

Based around their existing parts - 1u boxes etc punched for your connectors

Steel or aluminium podium plates

Much used pieces - typical parts I use; all from RS

Volume pots

Relays

Fancy LEDs

Multi-part illuminating switches - it's the holder that dictates latching or momentary, not the switch body.

Typical circuit fragments

Audio volume - passive for unbalanced and balanced

Audio pads.

Relay drives from low-voltage lines

Mains considerations - class 1?

Arduino - programmable board with analogue, digital (i.e. GPI & GPOs), a serial port and possibly Ethernet (network stack and good library).

Arduino cookbook

Video of the board, connecting it up etc

Some code examples

Some ideas for projects

The "enough already" video project

Section 2 - NetIOM and RaspberryPi

NetIOM - single board network control gadget, complete TCP/IP stack with GPIs, GPOs, analogue pins and a serial port.

Board is configured over an RS232 connection - remember to short/un-short the pin for programming. I put the short in the header of the serial cable so when the programming cable is plugged in the board is forced into programming mode.

Windows software.

Board can be interrogated over it's web interface.

Board can be configured to take action based on events.

Board can send email based on events

Two boards can be paired (hole opened in firewalls?) and the GPIs on one will mirror the GPOs on the other, same for the RS232 making it a very easy way of sending serial data over the Internet.

Thermistor can be used as a cheap temperature probe and the board can be configured to send an email when a certain temperature is reached and then close a relay when another temperature is reached; server room monitoring etc.

RaspberryPi

Single board Linux computer that boots off an SD card and runs from a 5v supply.

Network, HDMI and audio allow for a very cheap project platform; the may well the a software solution to what you're trying to achieve.